## 04/02/2026: in the PL room :

- Basics of a Photoluminescence setup (here, Hyperspectral imaging  « PL cube » IPL(x,y,λ)  in continuous regime (continuous laser) )

- Laser alignement

- PL acquisition (software PhySpec)

More precisely : objective lens : Nikon 50x 0.60 NA

Filters : BSW 26% Reflection + FELH 550nm

Laser : CW 532 nm

+ absolute calibration lamp (Bentham lamp)  spectra in photon number (and not arbitrarily camera counts)

## 11/02/2026 : analysis of PL data

PL cube :

IPL(λ, x, y)

We only look at the few pixels in the center of the excitation spot.

How do we extract information on hot carriers from a PL spectrum ?

- Temperature T  &  density n

Here : a spectrum = a photon distribution ; but this is related to T and n with the generalized Planck Law

IPLph.s-1.m-2.J-1.st-1=2E2h3c2A(E)expE-ΔμkBT-1

Actually, from (Δμ,T) we can retrieve the carrier density n.

## From (Δμ,T) to (n,T)

n=Eg/2∞dEeρeEe1expEe-μekBT +1

- Fermi-Dirac distribution (for electrons here) ; ρe is the electron density of states (you know its expression in the parabolic band approximation)

How do we relate μe to Δμ ?

Definition of Δμ=μe+μh   (in our convention)

We would like to have μe as function of Δμ only. To do so, we use the electroneutrality relation which stipulates

ne=nh    μh(μe,T)

Using the last two equations, we can compute Δμ(μe,T)=μe+μh(μe,T).

Finally we inverse the last function to obtain μe(Δμ,T).

Illustration in the low density regime (non-degenerate regime  /  Maxwell-Boltzmann approximation) :

n=Eg/2∞dEeρeEeexp-Ee-μekBT 	 we can compute this analytically

If μe+kBT≪Eg/2.

The formula is

ne=mekBT213πℏ232expμe-Eg/2kBT

Exercice : find explicitly μe(Δμ,T) within the Maxwell-Boltzmann distribution. How would you proceed without this approximation ?

## PL fit

Now, how do we extract (Δμ,T) from the spectrum, in practice ?  a fit

We don’t know the absorptivity AE !  (we could access it experimentally, or via simulation etc… but we will go to the simplest)

- make an assumption on A(E)

Above some energy Elower_bound we will consider that A(E)≈A0 (constant). So we will fit the GPL to the experimental points in a « high-energy » domain.

This approximation depends on a few things : material, structure (thickness  Beer Lambert law)   (carefull : the absorptivity here is only the absorptivity of the active layer)

We can estimate this A0 from an indendent optical measurement of the absorption coefficient.

It turns out that this value of the energy Elower_boundis significantly bigger than the bandgap Δμ+kBT (« high-energy » domain). So, we can further approximate the GPL (as long as we fit the law for E>Δμ+kBT):

IPLph.s-1.m-2.J-1.st-1=2E2h3c2A0exp-E-ΔμkBT

In practive, we will plot

logh3c22E2IPL=-E-ΔμkBT+logA0

As function of E

« High energy linear fit »

Slope   inverse of temperature

Intercept  chemical potential (substracted from absorptivity)

## 18/02/2026 :

Programm :

- error estimation

- correct the absorptivity

- relate excitation intensity to cooling power/intensity

Louis used to electroneutrality relation to access the µe, and µh (and n) in the Maxwell-Boltzmann approximation version. This overestimates by a tiny bit all those quantities, because, for a given Δμ, taking into account degeneracy decreases the occupation number (Pauli principle) and so the carrier number (/density).

Sources of error on physical parameters :

- statistical error on the minimization of chi²

- absorptivity at high energy A0

- fitting range : this was arbitrarily chosen, so to estimate the error coming from this, we try (for each excitation intensity) different fitting ranges  take the rms error of the ensemble of values found for each parameter

Estimation of the A0: simulation using OptiPV (IPVF tool for optical and electronical properties of well studied semiconductors)  from a table of the optical indexes (k,n) of GaAs the simulation returns the absorptivity of the sample (950nm thick, epitaxied on GaAs substrate), reflection and transmission could also be computed

The range we are interested in (high-energy range):

|  | Energy (eV) | Wavelength (nm) | Value of absorptivity |
| --- | --- | --- | --- |
| Lower bound | 1.50 | 827 | 0.459 |
| Upper bound | 1.70 | 729 | 0.555 |

Example : total error on photon chemical potential Δμ:

ΔΔμ=ΔΔμchi22+ΔΔμrange2+kBTA0ΔA02

From excitation power to thermalized / cooling / thermal intensity

As an exercise for Oussama:

- write the detailed balance equation in terms of carrier number and energy (consider only non-radiative recombination first)

- relate the absorbed flux to the absorbed power, same for recombination

- relate the thermalized power to the absorbed power (and to finish, to the excitation power)

what changes if radiative recombination aren’t negligible?

Why do we need this relation?

To compare experiment and theory (see Tsai article), more exactly, the comparison point is

Pth(n,T)

Next step : simulate this function from the microscopic model developed is Tsai’s article (in order to compare with experiment)

Reminder : d=950 nm the thickness of the GaAs active layer

Alas=0.625 the absorptivity at the laser wavelength

## 25/02/2026 :

Tsai’s model does the following :

Tsai’s simulation

Δμ, T					dudtintra=Pthd

(or μe or μh or n)

Carefull : here Pth is an intensity (W/cm²) not a power ; dividing it by the thickness makes it a power density (per unit volume)

Take the experimental points (Δμ, T) and use the simulation to predict the corresponding value of Pth, and finally compare it to the experimental value  actually we’ll do it the “other way around”

We take the experimental Δμ and Pth, and predict with the simulation the value of T, and compare it to the experimental points.

Do the following:

- write in python file that computes dudtintra  compute the electron contribution only (we will discuss that next time)

- try it on a few values for Δμ, T, or μe, T (but you know how to relate the µ’s)

- then, compute dudtintra for “a lot of” μe, T (continuous values)  table, register it, reverse it to have T(Pth,μe), interpolate the latter

- plot the experimental points with the simulated T(Pth,μe) evaluated at the experimental points (Pth,μe)